Value added processing of wheat bran for high soluble fiber production

Qingbin Guo1, Yikai Ren1, Douglas Goff1 and Steve W. Cui2

1Department of Food Science, University of Guelph, Guelph, Ontario N1G 2W1Canada

2Guelph Research and Development Centre, Agriculture and Agri-Food Canada, Guelph, Ontario N1G 5C9 Canada

Wheat bran1, one of the underutilized agricultural processing by-products, is rich in dietary fibers. However, the high percentage of insoluble dietary fiber hindered its food application as it compromised both the mouthfeel and health benefit (e.g. Type II diabetes) of the food products compared to soluble dietary fiber. Therefore, increasing the soluble fiber content brings more value added opportunity for wheat bran.

In our study, Alkaline (0.5 M and 1M, material to solution ratio of 1:1 w/v ) and cellulase (1unit/mL, material to solution ratio of 1:1 w/v) pre-treated wheat bran were subjected to extrusion process through a laboratory-scale conical corotating and intermeshing twin-screw extruder with the moisture content of 40% (L/D ratio of 10:1; 4 mm cicular die; Plasti-corder Digi-system, PL 2200, Brabender Instruments, Inc South Hackensack, NJ). SEM analysis indicated that pre-treatment and extrusion process has largely ruptured and distorted the cell wall structure. After treatment, the soluble fiber content has been significantly increased (P<0.05) for both alkaline and cellulose treated samples. This was because most of the previously bounded hemicellulose has been released after treatment. HPSEC analysis indicated a similar molecular weight distribution of hemicellulose before and after treatment. In addition, using hot water soaking, the alkaline treated wheat bran can be easily separated into three fractions: cellulose rich fraction (hot water insoluble fraction), lignin rich fraction (hot water soluble, but not insoluble after neutralization) and hemicellulose rich fractions (soluble fiber, mainly arabinoxylan, soluble in the solution after neutralization).

Therefore, the novel processed wheat bran can be either incorporated for high soluble fiber food products or used as a source material to generate cellulose, lignin and hemicellulose rich ingredients. For the industrial application, the use of treated wheat bran as binding agents and stabilisers for aggragated pathway surfaces and for the joints of interlocking paving stones have also been considered in our research.

References:

1	Stevenson, L., Phillips, F., O’sullivan, K., & Walton, J. (2012). Wheat bran: its composition and benefits to health, a European perspective. International Journal of Food Sciences and Nutrition, 63(8), 1001–1013. http://doi.org/10.3109/09637486.2012.687366